Chapter 8 Partial Differential Equations

8-1 Elliptic Partial Differential Equations

Solve ∂2u(x,y)∂x2+∂2u(x,y)∂y2=f(x,y), ∀(x,y)∈R; u(x,y)=g(x,y), ∀(x,y)∈S, where R={(x,y): a≦x≦b, c≦y≦d}, and S is the boundary of R.

Set xi=a+ih, yj=c+jk, h=(b-a)/n, k=(d-c)/m, i=0, 1, 2, …, n, and j=0, 1, 2, …, m.

Central difference method:

u(xi+1,yj)−2u(xi,yj)+u(xi−1,yj)h2+u(xi,yj+1)−2u(xi,yj)+u(xi,yj−1)k2=f(xi,yj)

and u(x0,yj)=g(x0,yj), u(xn,yj)=g(xn,yj), j=0, 1, 2, …, m-1; u(xi,y0)=g(xi,y0), u(xi,ym)=g(xi,ym), i=0, 1, 2, …, n-1

⇒2hk2+1uij−(ui+1,j+ui−1,j)−hk2(ui,j+1+ui,j−1)=−h2fij and u0j=g0j, unj=gnj, ui0=gi0, uim=gim.

Eg. Solve ∂2u∂x2+∂2u∂y2=0, 0≦x≦0.5, 0≦y≦0.5 with u(0,y)=u(x,0)=0, u(x,1/2)=200x, u(1/2,y)=200y. (Exact solution: u(x,y)=400xy)

(Sol.) We set n=m=4, and then

h=k=124=18, uij=u(xi,yj), xi=ihyj=jk⇒ui+1,j−2ui,j+ui−1,jh2+ui,j+1−2ui,j+ui,j−1k2=0

⇒2hk2+1uij−(ui+1,j+ui−1,j)−hk2(ui,j+1+ui,j−1)=0

⇒4u13−u23−u12=u14=200×18=254u23−u33−u13−u22=u24=200×28=504u33−u23−u32=u43+u34=200×38+200×38=1504u12−u22−u13−u11=u02=04u22−u32−u12−u23−u21=04u32−u22−u33−u31=u42=200×28=504u11−u21−u12=u01+u10=04u21−u31−u11−u22=u20=0⇒4−10−100000−14−10−100000−1400−1000−1004−10−1000−10−14−10−1000−10−1400−1000−1004−100000−10−14−100000−10−14 u13u23u33u12u22u32u11u21u31=255015000500025

⇒u13=18.75, u23=37.5, …, u21=12.5, u31=18.75

In MATLAB language, we can use the following program to solve the partial differential equation:

>>A=[4,-1,0,-1,0,0,0,0,0;-1,4,-1,0,-1,0,0,0,0;0,-1,4,0,0,-1,0,0,0;-1,0,0,4,-1,0,-1,0,0;

0,-1,0,-1,4,-1,0,-1,0;0,0,-1,0,-1,4,0,0,-1;0,0,0,-1,0,0,4,-1,0;0,0,0,0,-1,0,-1,4,-1;

0,0,0,0,0,-1,0,-1,4];

>>B=[25;50;150;0;0;50;0;0;25];

>>rref([A,B])

ans =

Columns 1 through 7

1.0000         0         0         0         0         0         0

0    1.0000         0         0         0         0         0

0         0    1.0000         0         0         0         0

0         0         0    1.0000         0         0         0

0         0         0         0    1.0000         0         0

0         0         0         0         0    1.0000         0

0         0         0         0         0         0    1.0000

0         0         0         0         0         0         0

0         0         0         0         0         0         0

Columns 8 through 10

0         0   18.7500

0         0   37.5000

0         0   56.2500

0         0   12.5000

0         0   25.0000

0         0   37.5000

0         0    6.2500

1.0000         0   12.5000

0    1.0000   18.7500

8-2 Parabolic Partial Differential Equations

Solve ∂u(x,t)∂t=α2⋅∂2u(x,t)∂x2, 0≦x≦l, t>0, with u(0,t)=u(l,t)=0, u(x,0)=f(x).

Set h=l/m, xi=ih, tj=jk, i=0, 1, 2, …, m, and j=0, 1, 2, …

Forward difference method: ui,j+1−uijk−α2⋅ui+1,j−2uij+ui−1,jh2=0

Set λ=α2kh2⇒ui,j+1=(1−2λ)uij+λ(ui+1,j+ui−1,j) and u0j=umj=0, ui0=f(xi)

⇒u1,j+1u2,j+1⋮⋮um−1,j+1=(1−2λ)λ0⋯0λ(1−2λ)λ⋱⋮0λ(1−2λ)⋱0⋮⋱⋱⋱λ0⋯0λ(1−2λ) u1ju2j⋮⋮um−1,j

We have

u11u21⋮⋮um−1,1=(1−2λ)λ0⋯0λ(1−2λ)λ⋱⋮0λ(1−2λ)⋱0⋮⋱⋱⋱λ0⋯0λ(1−2λ) f(x1)f(x2)⋮⋮f(xm−1),

u12u22⋮⋮um−1,2=(1−2λ)λ0⋯0λ(1−2λ)λ⋱⋮0λ(1−2λ)⋱0⋮⋱⋱⋱λ0⋯0λ(1−2λ) u11u21⋮⋮um−1,1,

u13u23⋮⋮um−1,3=(1−2λ)λ0⋯0λ(1−2λ)λ⋱⋮0λ(1−2λ)⋱0⋮⋱⋱⋱λ0⋯0λ(1−2λ) u12u22⋮⋮um−1,2,u14u24⋮⋮um−1,4=(1−2λ)λ0⋯0λ(1−2λ)λ⋱⋮0λ(1−2λ)⋱0⋮⋱⋱⋱λ0⋯0λ(1−2λ) u13u23⋮⋮um−1,3, ….

Backward difference method: uij−ui,j−1k−α2⋅ui+1,j−2uij+ui−1,jh2=0

⇒ui,j−1=(1+2λ)uij−λ(ui+1,j+ui−1,j)

(1+2λ)−λ0⋯0−λ(1+2λ)−λ⋱⋮0⋱⋱⋱0⋮⋱⋱⋱−λ0⋯0−λ(1+2λ) u1ju2j⋮⋮um−1,j=u1,j−1u2,j−1⋮⋮um−1,j−1, where ui0=f(xi).

We solve

(1+2λ)−λ0⋯0−λ(1+2λ)−λ⋱⋮0⋱⋱⋱0⋮⋱⋱⋱−λ0⋯0−λ(1+2λ) u11u21⋮⋮um−1,1=f(x1)f(x2)⋮⋮f(xm−1)⇒ u11u21⋮⋮um−1,1=?

and then we solve

(1+2λ)−λ0⋯0−λ(1+2λ)−λ⋱⋮0⋱⋱⋱0⋮⋱⋱⋱−λ0⋯0−λ(1+2λ) u12u22⋮⋮um−1,2=u11u21⋮⋮um−1,1⇒ u12u22⋮⋮um−1,2=?

and then we solve

(1+2λ)−λ0⋯0−λ(1+2λ)−λ⋱⋮0⋱⋱⋱0⋮⋱⋱⋱−λ0⋯0−λ(1+2λ) u13u23⋮⋮um−1,3=u12u22⋮⋮um−1,2⇒ u13u23⋮⋮um−1,3=?

and then we solve

(1+2λ)−λ0⋯0−λ(1+2λ)−λ⋱⋮0⋱⋱⋱0⋮⋱⋱⋱−λ0⋯0−λ(1+2λ) u14u24⋮⋮um−1,4=u13u23⋮⋮um−1,3⇒ u14u24⋮⋮um−1,4=?

, ….

Eg. Solve ∂u(x,t)∂t−∂2u(x,t)∂x2=0, 0≦x≦1, t>0 with u(0,t)=u(1,t)=0, and u(x,0)=sin(πx).

(Sol.) Use the Backward-difference method, and choose m=10, h=0.1, k=0.01⇒λ=1, 1+2λ=3, xi=0.1i, tj=0.01j, and then we have

3−10⋯0−13−1⋱⋮0⋱⋱⋱0⋮⋱⋱⋱−10⋯0−13 u11u21⋮⋮u91= u10u20⋮⋮u90=sin(πx1)sin(πx2)⋮⋮sin(πx9)

⇒ u11u21⋮⋮u91=3−10⋯0−13−1⋱⋮0⋱⋱⋱0⋮⋱⋱⋱−10⋯0−13−1 sin(πx1)sin(πx2)⋮⋮sin(πx9), and

3−10⋯0−13−1⋱⋮0⋱⋱⋱0⋮⋱⋱⋱−10⋯0−13 u12u22⋮⋮u92=u11u21⋮⋮u91⇒ u12u22⋮⋮u92=3−10⋯0−13−1⋱⋮0⋱⋱⋱0⋮⋱⋱⋱−10⋯0−13−1u11u21⋮⋮u91,

3−10⋯0−13−1⋱⋮0⋱⋱⋱0⋮⋱⋱⋱−10⋯0−13 u13u23⋮⋮u93=u12u22⋮⋮u92⇒ u13u23⋮⋮u93=3−10⋯0−13−1⋱⋮0⋱⋱⋱0⋮⋱⋱⋱−10⋯0−13−1u12u22⋮⋮u92,

3−10⋯0−13−1⋱⋮0⋱⋱⋱0⋮⋱⋱⋱−10⋯0−13 u14u24⋮⋮u94=u13u23⋮⋮u93⇒ u14u24⋮⋮u94=3−10⋯0−13−1⋱⋮0⋱⋱⋱0⋮⋱⋱⋱−10⋯0−13−1u13u23⋮⋮u93,

…

In Matlab language, we can use the following program to solve the partial differential equation:

>> A=[3,-1,0,0,0,0,0,0,0;-1,3,-1,0,0,0,0,0,0;0,-1,3,-1,0,0,0,0,0;0,0,-1,3,-1,0,0,0,0;

0,0,0,-1,3,-1,0,0,0;0,0,0,0,-1,3,-1,0,0;0,0,0,0,0,-1,3,-1,0;0,0,0,0,0,0,-1,3,-1;

0,0,0,0,0,0,0,-1,3];

>>pi=3.1415926;

>>B=[sin(0.1*pi);sin(0.2*pi);sin(0.3*pi);sin(0.4*pi);sin(0.5*pi);sin(0.6*pi);sin(0.7*pi);sin(0.8*pi);sin(0.9*pi)];

>>for i=1:5

C=inv(A)*B

B=C;

end

C =

0.1765

0.3356

0.4620

0.5431

0.5710

0.5431

0.4620

0.3356

0.1765

C =

0.1607

0.3057

0.4208

0.4947

0.5201

0.4947

0.4208

0.3057

0.1607

C =

0.1464

0.2785

0.3833

0.4506

0.4737

0.4506

0.3833

0.2785

0.1464

C =

0.1333

0.2536

0.3491

0.4104

0.4315

0.4104

0.3491

0.2536

0.1333

C =

0.1215

0.2310

0.3180

0.3738

0.3930

0.3738

0.3180

0.2310

0.1215

Richardson’s method: ui,j+1−ui,j−12k−α2ui+1,j−2uij+ui−1,jh2=0.

Note: Forward difference method is conditional stable. Richardson’s method has a serious stability problem.

Crank-Nicholson method: Au(j+1)=Bu(j)

(1+λ)−λ20⋯0−λ2(1+λ)−λ2⋱⋮0⋱⋱⋱0⋮⋱⋱⋱−λ20⋯0−λ2(1+λ)u1,j+1u2,j+1⋮⋮⋮um−1,j+1=(1−λ)λ20⋯0λ2(1−λ)λ2⋱⋮0⋱⋱⋱0⋮⋱⋱⋱λ20⋯0λ2(1−λ) u1ju2j⋮⋮um−1,j, where ui0=f(xi).

We solve

(1+λ)−λ20⋯0−λ2(1+λ)−λ2⋱⋮0⋱⋱⋱0⋮⋱⋱⋱−λ20⋯0−λ2(1+λ)u11u21⋮⋮⋮um−1,1=(1−λ)λ20⋯0λ2(1−λ)λ2⋱⋮0⋱⋱⋱0⋮⋱⋱⋱λ20⋯0λ2(1−λ) f(x1)f(x2)⋮⋮f(xm−1),

and then we solve

(1+λ)−λ20⋯0−λ2(1+λ)−λ2⋱⋮0⋱⋱⋱0⋮⋱⋱⋱−λ20⋯0−λ2(1+λ)u12u22⋮⋮⋮um−1,2=(1−λ)λ20⋯0λ2(1−λ)λ2⋱⋮0⋱⋱⋱0⋮⋱⋱⋱λ20⋯0λ2(1−λ) u11u21⋮⋮um−1,1,

and then we solve

(1+λ)−λ20⋯0−λ2(1+λ)−λ2⋱⋮0⋱⋱⋱0⋮⋱⋱⋱−λ20⋯0−λ2(1+λ)u13u23⋮⋮⋮um−1,3=(1−λ)λ20⋯0λ2(1−λ)λ2⋱⋮0⋱⋱⋱0⋮⋱⋱⋱λ20⋯0λ2(1−λ) u12u22⋮⋮um−1,2,

and then we solve

(1+λ)−λ20⋯0−λ2(1+λ)−λ2⋱⋮0⋱⋱⋱0⋮⋱⋱⋱−λ20⋯0−λ2(1+λ)u14u24⋮⋮⋮um−1,4=(1−λ)λ20⋯0λ2(1−λ)λ2⋱⋮0⋱⋱⋱0⋮⋱⋱⋱λ20⋯0λ2(1−λ) u13u23⋮⋮um−1,3,

…

8-3 Hyperbolic Partial Differential Equations

Solve ∂2u(x,t)∂t2−α2∂2u(x,t)∂x2=0, 0≦x≦l, t>0 with u(0,t)=u(l,t)=0, u(x,0)=f(x), ∂u∂ t(x,0)=g(x).

Central difference method: Set h=l/m, xi=ih, tj=jk, and λ=αk/h

⇒ui,j+1−2uij+ui,j−1k2−α2⋅ui+1,j−2uij+ui−1,jh2=0⇒ui,j+1=2(1−λ2)uij+λ2(ui+1,j+ui−1,j)−ui,j−1, ui0=f(xi), u0j=umj=0, and ui1=ui0+kg(xi)(1−λ2)ui0+λ22[f(xi+1)+f(xi−1)]+kg(xi)

⇒u1,j+1u2,j+1⋮⋮um−1,j+1=2(1−λ2)λ20⋯0λ22(1−λ2)λ2⋱⋮0⋱⋱⋱0⋮⋱⋱⋱λ20⋯0λ22(1−λ2) u1ju2j⋮⋮um−1,j−u1,j−1u2,j−1⋮⋮um−1,j−1

We have

u12u22⋮⋮um−1,2=2(1−λ2)λ20⋯0λ22(1−λ2)λ2⋱⋮0⋱⋱⋱0⋮⋱⋱⋱λ20⋯0λ22(1−λ2) u11u21⋮⋮um−1,1−f(x1)f(x2)⋮⋮f(xm−1),

u13u23⋮⋮um−1,3=2(1−λ2)λ20⋯0λ22(1−λ2)λ2⋱⋮0⋱⋱⋱0⋮⋱⋱⋱λ20⋯0λ22(1−λ2) u12u22⋮⋮um−1,2−u11u21⋮⋮um−1,1,

u14u24⋮⋮um−1,4=2(1−λ2)λ20⋯0λ22(1−λ2)λ2⋱⋮0⋱⋱⋱0⋮⋱⋱⋱λ20⋯0λ22(1−λ2) u13u23⋮⋮um−1,3−u12u22⋮⋮um−1,2, …

Eg. Solve ∂2u(x,t)∂ t2−4∂2u(x,t)∂x2=0, 0≦x≦1, t>0 with u(0,t)=u(1,t)=0, u(x,0)=sin(πx), and ∂u(x,0)∂ t=0.

(Sol.) Set m=10, h=0.1, k=0.05⇒λ=1, λ2=1, 2(1-λ2)=0, xi=0.1i, tj=0.05j

⇒ui0=f(xi)=sin(πxi), u0j=umj=0,

ui1≈12[f(xi+1)+f(xi−1)]+0.05g(xi)=12[sin(πxi+1)+sin(πxi−1)],

and then we have  u10u20⋮⋮u90=sin(πx1)sin(πx2)⋮⋮sin(πx9),  u11u21⋮⋮u91=12[sin(πx2)+sin(πx0)]12[sin(πx3)+sin(πx1)]⋮⋮12[sin(πx10)+sin(πx8)],

u12u22⋮⋮u92=010⋯0101⋱⋮0⋱⋱⋱0⋮⋱⋱⋱10⋯010 12[sin(πx2)+sin(πx0)]12[sin(πx3)+sin(πx1)]⋮⋮12[sin(πx10)+sin(πx8)]-sin(πx1)sin(πx2)⋮⋮sin(πx9)

⇒u13u23⋮⋮u93=010⋯0101⋱⋮0⋱⋱⋱0⋮⋱⋱⋱10⋯010 u12u22⋮⋮u92-12[sin(πx2)+sin(πx0)]12[sin(πx3)+sin(πx1)]⋮⋮12[sin(πx10)+sin(πx8)]

⇒u14u24⋮⋮u94=010⋯0101⋱⋮0⋱⋱⋱0⋮⋱⋱⋱10⋯010 u13u23⋮⋮u93−u12u22⋮⋮u92

⇒u15u25⋮⋮u95=010⋯0101⋱⋮0⋱⋱⋱0⋮⋱⋱⋱10⋯010 u14u24⋮⋮u94−u13u23⋮⋮u93⇒…

In Matlab language, we can use the following program to solve the partial differential equation:

>>A=[0,1,0,0,0,0,0,0,0;1,0,1,0,0,0,0,0,0;0,1,0,1,0,0,0,0,0;0,0,1,0,1,0,0,0,0;

0,0,0,1,0,1,0,0,0;0,0,0,0,1,0,1,0,0;0,0,0,0,0,1,0,1,0;0,0,0,0,0,0,1,0,1;

0,0,0,0,0,0,0,1,0];

>>pi=3.1415926;

>>B=[sin(0.1*pi);sin(0.2*pi);sin(0.3*pi);sin(0.4*pi);sin(0.5*pi);sin(0.6*pi);sin(0.7*pi);sin(0.8*pi);sin(0.9*pi)];

>>C=[(0+sin(0.2*pi))/2;(sin(0.1*pi)+sin(0.3*pi))/2;(sin(0.2*pi)+sin(0.4*pi))/2;(sin(0.3*pi)+sin(0.5*pi))/2;(sin(0.4*pi)+sin(0.6*pi))/2;(sin(0.5*pi)+sin(0.7*pi))/2;(sin(0.6*pi)+sin(0.8*pi))/2;(sin(0.7*pi)+sin(0.9*pi))/2;(sin(0.8*pi)+sin(pi))/2];

>>for i=2:5

D=A*C-B

B=C;

C=D;

end

D =

0.2500

0.4755

0.6545

0.7694

0.8090

0.7694

0.6545

0.4755

0.2500

D =

0.1816

0.3455

0.4755

0.5590

0.5878

0.5590

0.4755

0.3455

0.1816

D =

0.0955

0.1816

0.2500

0.2939

0.3090

0.2939

0.2500

0.1816

0.0955

D =

1.0e-007 *

0.0828

0.1575

0.2168

0.2548

0.2679

-0.2548

-0.2168

-0.1575

-0.0828

8-4 Finite-Difference Time-Domain (FDTD) Method of Solving Maxwell’s Equations

3D Full-vector FDTD method (by Kane S. Yee, 1966):

Maxwell’s equations in the source-free region: ∇×H=ε∂E∂t, ∇×E=μ∂H∂t

⇒ε∂Ex∂t=∂Hz∂y−∂Hy∂z, ε∂Ey∂t=∂Hx∂z−∂Hz∂x, ε∂Ez∂t=∂Hy∂x−∂Hx∂y,

μ∂Hx∂t=∂Ey∂z−∂Ez∂y, μ∂Hy∂t=∂Ez∂x−∂Ex∂z, μ∂Hz∂t=∂Ex∂y−∂Ey∂x

(Yee’s cell)

Define the following notations: E(i△x,j△y,k△z,l△t)≡E(i,j,k,l)≡El(i,j,k) and H(i△x,j△y,k△z,l△t)≡H(i,j,k,l)≡Hl(i,j,k). According to Yee’s cell, we have

Ex≡Ex(i+1/2,j,k,l)≡Exl(i+1/2,j,k),

Ey≡Ey(i,j+1/2,k,l)≡Eyl(i,j+1/2,k),

Ez≡Ez(i,j,k+1/2,l)≡Ezl(i,j,k+1/2),

Hx≡Hx(i,j+1/2,k+1/2,l-1/2)≡Hxl-1/2(i,j+1/2,k+1/2),

Hy≡Hy(i+1/2,j,k+1/2,l-1/2)≡Hyl-1/2(i+1/2,j,k+1/2),

Hz≡Hz(i+1/2,j+1/2,k,l-1/2)≡Hz l-1/2(i+1/2,j+1/2,k),

∂Ex∂t≈Ex(i+12,j,k,l+1)−Ex(i+12,j,k,l)Δt,

∂Ey∂t≈Ey(i,j+12,k,l+1)−Ey(i,j+12,k,l)Δt,

∂Ez∂t≈Ez(i,j,k+12,l+1)−Ez(i,j,k+12,l)Δt,

∂Ex∂y≈Ex(i+12,j,k,l)−Ex(i+12,j−1,k,l)Δy,

∂Ex∂z≈Ex(i+12,j,k,l)−Ex(i+12,j,k−1,l)Δz,

∂Ey∂x≈Ey(i,j+12,k,l)−Ey(i−1,j+12,k,l)Δx,

∂Ey∂z≈Ey(i,j+12,k,l)−Ey(i,j+12,k−1,l)Δz,

∂Ez∂x≈Ez(i,j,k+12,l)−Ez(i−1,j,k+12,l)Δx,

∂Ez∂y≈Ez(i,j,k+12,l)−Ez(i,j−1,k+12,l)Δy,

But

∂Hx∂t≈Hx(i,j+12,k+12,l+12)−Hx(i,j+12,k+12,l−12)Δt,

∂Hy∂t≈Hy(i+12,j,k+12,l+12)−Hy(i+12,j,k+12,l−12)Δt,

∂Hz∂t≈Hz(i+12,j+12,k,l+12)−Hz(i+12,j+12,k,l−12)Δt,

∂Hx∂y≈Hx(i,j+12,k+12,l−12)−Hx(i,j−12,k+12,l−12)Δy,

∂Hx∂z≈Hx(i,j+12,k+12,l−12)−Hx(i,j+12,k−12,l−12)Δz,

∂Hy∂x≈Hy(i+12,j,k+12,l−12)−Hy(i−12,j,k+12,l−12)Δx,

∂Hy∂z≈Hy(i+12,j,k+12,l−12)−Hy(i+12,j,k−12,l−12)Δz,

∂Hz∂x≈Hz(i+12,j+12,k,l−12)−Hz(i−12,j+12,k,l−12)Δx,

∂Hz∂y≈Hz(i+12,j+12,k,l−12)−Hz(i+12,j−12,k,l−12)Δy

⇒Exl+1(i+12,j,k)=Exl(i+12,j,k)+C(i+12,j,k)⋅Hzl−12(i+12,j+12,k)−Hzl−12(i+12,j−12,k)Δy−Hyl−12(i+12,j,k+12)−Hyl−12(i+12,j,k−12)Δz,

Eyl+1(i,j+12,k)=Eyl(i,j+12,k)+C(i,j+12,k)⋅Hxl−12(i,j+12,k+12)−Hxl+12(i,j+12,k−12)Δz−Hzl−12(i+12,j+12,k)−Hzl−12(i−12,j+12,k)Δx,

Ezl+1(i,j,k+12)=Ezl(i,j,k+12)+C(i,j,k+12)⋅Hyl−12(i+12,j,k+12)−Hyl−12(i−12,j,k+12)Δx−Hxl−12(i,j+12,k+12)−Hxl−12(i,j−12,k+12)Δy,

Hxl+12(i,j+12,k+12)=Hxl−12(i,j+12,k+12)+D(i,j+12,k+12)⋅−Ezl(i,j+1,k+12)−Ezl(i,j,k+12)Δy+Eyl(i,j+12,k+1)−Eyl(i,j+12,k)Δz,

Hyl+12(i+12,j,k+12)=Hyl−12(i+12,j,k+12)+D(i+12,j,k+12)⋅Ezl(i+1,j,k+12)−Ezl(i,j,k+12)Δx−Exl(i+12,j,k+1)−Exl(i+12,j,k)Δz,

Hzl+12(i+12,j+12,k)=Hzl−12(i+12,j+12,k)+D(i+12,j+12,k)⋅Exl(i+12,j+1,k)−Exl(i+12,j,k)Δy−Eyl(i+1,j+12,k)−Eyl(i,j+12,k)Δx,

where C(i,j,k)=Δtε(i,j,k),  D(i,j,k)=Δtμ(i,j,k), and the electromagnetic power distribution E2=(Ex2+Ey2+Ez2).

Stability criterion: Δt≤1vmax1Δx2+1Δy2+1Δz2, where vmax is the maximal velocity of EM wave propagating.

3D Scalar FDTD method (by Dr. Wei-Ping Huang, 1991):

Dr. Wei-Ping Huang was born in Beijing. He graduated from MIT and got his PhD degree. Now he is a professor in the ECE Department at the University of Waterloo, Canada. His research group is one of the leading groups internationally in photonics and computer aided design. Dr. Huang is creating new design tools to help develop the highly-efficient photonics systems required for large-scale broadband communication.

Consider a wave equation: ∇2Ψ+n2c2∂2Ψ∂t2=∂2Ψ∂x2+∂2Ψ∂y2+∂2Ψ∂z2+n2c2∂2Ψ∂t2=0, where Ψ(x,y,z,t) is the scalar field (approximation) and n(x,y,z) is the refractive index. Using Ψ(i△x,j△y,k△z,l△t)≡Ψ(i,j,k,l)≡Ψl(i,j,k), where 0≦i≦I, 0≦j≦J, and 0≦k≦K.

Let ∂2Ψ∂x2=Ψl(i+1,j,k)−2Ψl(i,j,k)+Ψl(i−1,j,k)Δx2,

∂2Ψ∂y2=Ψl(i,j+1,k)−2Ψl(i,j,k)+Ψl(i,j−1,k)Δy2,

∂2Ψ∂z2=Ψl(i,j,k+1)−2Ψl(i,j,k)+Ψl(i,j,k−1)Δz2,

and ∂2Ψ∂t2=Ψl+1(i,j,k)−2Ψl(i,j,k)l+Ψl−1(i,j,k)Δt2

⇒Ψl+1(i,j,k)=2⋅[1−δx2+δy2+δz2n2(i,j,k)]⋅Ψl(i,j,k)−Ψl−1(i,j,k)

+δx2n2(i,j,k)⋅[Ψl(i+1,j,k)+Ψl(i−1,j,k)]+δy2n2(i,j,k)⋅[Ψl(i,j+1,k)+Ψl(i,j−1,k)]

+δz2n2(i,j,k)⋅[Ψl(i,j,k+1)+Ψl(i,j,k−1)]

where δx=cΔtΔx, δy=cΔtΔy, δz=cΔtΔz.

The electromagnetic power distribution  |Ψ|2.

The 3D scalar FDTD method may be incorporated with the Mur’s absorbing boundary conditions.

Mur’s First-Order Absorbing Boundary Conditions:

At x=0, Ψl+1(0,j,k)=Ψl(1,j,k)+cΔt−ΔxcΔt+Δx⋅[Ψl+1(1,j,k)−Ψl(0,j,k)]

x=I△x, Ψl+1(I,j,k)=Ψl(I−1,j,k)+cΔt−ΔxcΔt+Δx⋅[Ψl+1(I−1,j,k)−Ψl(I,j,k)]

y=0, Ψl+1(i,0,k)=Ψl(i,1,k)+cΔt−ΔycΔt+Δy⋅[Ψl+1(i,1,k)−Ψl(i,0,k)]

y=J△y, Ψl+1(i,J,k)=Ψl(i,J−1,k)+cΔt−ΔycΔt+Δy⋅[Ψl+1(i,J−1,k)−Ψl(i,J,k)]

z=0, Ψl+1(i,j,0)=Ψl(i,j,0)+cΔt−ΔzcΔt+Δz⋅[Ψl+1(i,j,1)−Ψl(i,j,0)]

z=K△z, Ψl+1(i,j,K)=Ψl(i,j,K−1)+cΔt−ΔzcΔt+Δz⋅[Ψl+1(i,j,K−1)−Ψl(i,j,K)]

3D Semi-vectorial FDTD method (also by Dr. Wei-Ping Huang):

Exl+1(i,j,k)=2⋅1−[2−T(i+1,j,k)+T(i−1,j,k)2]⋅δx2+δy2+δz2n2(i,j,k)⋅Exl(i,j,k)−Exl−1(i,j,k)

+δx2n2(i,j,k)⋅[T(i+1,j,k)⋅Exl(i+1,j,k)+T(i−1,j,k)⋅Exl(i−1,j,k)]

+δy2n2(i,j,k)⋅[Exl(i,j+1,k)+Exl(i,j−1,k)]+δz2n2(i,j,k)⋅[Exl(i,j,k+1)+Exl(i,j,k−1)]

Eyl+1(i,j,k)=2⋅1−δx2+[2−T(i,j+1,k)+T(i,j−1,k)2]⋅δy2+δz2n2(i,j,k)⋅Eyl(i,j,k)−Eyl−1(i,j,k)

+δx2n2(i,j,k)⋅[Eyl(i+1,j,k)+Eyl(i−1,j,k)]+δz2n2(i,j,k)⋅[Eyl(i,j,k+1)+Eyl(i,j,k−1)]

+δy2n2(i,j,k)⋅[T(i,j+1,k)⋅Eyl(i,j+1,k)+T(i,j−1,k)⋅Eyl(i,j−1,k)]

where T(i±1,j,k)=2n2(i±1,j,k)n2(i±1,j,k)+n2(i,j,k) and T(i,j±1,k)=2n2(i,j±1,k)n2(i,j±1,k)+n2(i,j,k).

The 3D Semi-vectorial FDTD method may be incorporated with the perfectly matched layer (PML) boundary conditions.

Perfectly matched layer (PML) boundary conditions:

In the x-direction:

Ψl+1i,j,k=−Ψl−1i,j,k+21−δy2+δz2n2i,j,kΨli,j,k+δx2n2i,j,kD2xl+12i,j,k−D2xl−12i,j,k+δy2n2i,j,kΨli,j+1,k+Ψli,j−1,k+δz2n2i,j,kΨli,j,k+1+Ψli,j,k−1, where δx=cΔtΔx, δy=cΔtΔy, δz=cΔtΔz

D1xl+1i+1,j,k=ax−σx⋅Δtε0n2i+1,j,kax+σx⋅Δtε0n2i+1,j,k⋅D1xl−1i+1,j,k

+1ax+σx⋅Δtε0n2i+1,j,k⋅Ψli+1,j,k−Ψli,j,k

D2xl+1i,j,k=ax−σx⋅Δtε0n2i+1,j,kax+σx⋅Δtε0n2i+1,j,k⋅D2xl−1i,j,k

+1ax+σx⋅Δtε0n2i+1,j,k⋅[D1xl+1i+1,j,k−D1xl+1i−1,j,k

−D1xl−1i+1,j,k+D1xl−1i−1,j,k],

σxx=xdmσxmax=−xdmnx2ε0cm+1lnR02d

d=nxΔx: thickness of PML, m: order, R0: reflection from PML at normal incidence

nx=8, R0=10−2, m=4

In the y-direction:

Ψl+1i,j,k=−Ψl−1i,j,k+21−δx2+δz2n2i,j,kΨli,j,k

+δx2n2i,j,kΨli+1,j,k+Ψli−1,j,k+δy2n2i,j,kD2yl+12i,j,k−D2yl−12i,j,k+δz2n2i,j,kΨli,j,k+1+Ψli,j,k−1,

where δx=cΔtΔx, δy=cΔtΔy, δz=cΔtΔz

D1yl+1i,j+1,k=ay−σy⋅Δtε0n2i,j+1,kay+σy⋅Δtε0n2i,j+1,k⋅D1yl−1i,j+1,k

+1ay+σy⋅Δtε0n2i,j+1,k⋅Ψli,j+1,k−Ψli,j,k

D2yl+1i,j,k=ay−σy⋅Δtε0n2i,j+1,kay+σy⋅Δtε0n2i,j+1,k⋅D2yl−1i,j,k

+1ay+σy⋅Δtε0n2i,j+1,k⋅[D1yl+1i,j+1,k−D1yl+1i,j−1,k

−D1yl−1i,j+1,k+D1yl−1i,j−1,k],

σyy=ydmσymax=−ydmny2ε0cm+1lnR02d

d=nyΔy: thickness of PML, m: order

R0: reflection from PML at normal incidence

ny=8, R0=10−2, m=4

In the z-direction:

Ψl+1i,j,k=−Ψl−1i,j,k+21−δx2+δy2n2i,j,kΨli,j,k

+δx2n2i,j,kΨli+1,j,k+Ψli−1,j,k+δy2n2i,j,kΨli,j+1,k+Ψli,j−1,k+δz2n2i,j,kD2zl+12i,j,k−D2zl−12i,j,k,

where δx=cΔtΔx, δy=cΔtΔy, δz=cΔtΔz

D1zl+1i,j,k+1=az−σz⋅Δtε0n2i,j,k+1az+σz⋅Δtε0n2i,j,k+1⋅D1zl−1i,j,k+1

+1az+σz⋅Δtε0n2i,j,k+1⋅Ψli,j,k+1−Ψli,j,k

σzz=zdmσzmax=−zdmnz2ε0cm+1lnR02d

+1az+σz⋅Δt2ε0n2i,j,k+1⋅[D1zl+1i,j,k+1−D1zl+1i,j,k−1

−D1zl−1i,j,k+1+D1zl−1i,j,k−1]

σzz=zdmσzmax=−zdmnz2ε0cm+1lnR02d

d=nzΔz: thickness of PML, m: order

R0: reflection from PML at normal incidence

nz=8, R0=10−2, m=4

Eg. Use FDTD method to simulate the characteristics of photonic crystal T-junction waveguide. (by Y. -J. Lin, 林宇仁)

Eg. Use FDTD method to simulate the characteristics of photonic crystal Y-junction waveguide.

Eg. Use FDTD method to simulate the characteristics of LED.

8-5 Finite-difference Beam Propagation Method (FDBPM)

Consider that a light wave propagates along z-directional optical waveguide and set E=xE'x+yE'y+zE'z in a source-free region.

E-formulation (by Dr. Wei-Ping Huang):

∇×∇×E−n2k2E=∇(∇⋅E)−∇2E−n2k2E=0⇒∇2E+n2k2E=∇(∇⋅E)

⇒∇2(xE'x+yE'y+zE'z)+n2k2(xE'x+yE'y+zE'z)=∇(∂E'x∂x+∂E'y∂y+∂E'z∂z)

∵ No source, ∴∇⋅(εE)=∇⋅(n2E)=0⇒∂(n2E'x)∂x+∂(n2E'y)∂y+∂n2∂zE'z+n2∂E'z∂z=0

If the refractive index profile varies slowly along the z-axis, ∂n2/∂z may be neglected. ⇒∂E'z∂z≈−1n2[∂(n2E'x)∂x+∂(n2E'y)∂y]

∇2(xE'x+yE'y+zE'z)+n2k2(xE'x+yE'y+zE'z)(x∂∂x+y∂∂y+z∂∂z)(∂E'x∂x+∂E'y∂y−1n2[∂(n2E'x)∂x+∂(n2E'y)∂y])

⇒∇2E'x+n2k2E'x=∂∂x,

and ∇2E'y+n2k2E'y=∂∂y.

Assume that E'x=Exe−jn0kz and E'y=Eye−jn0kz, where k is the vacuum wave number and n0=β/k is the effective refractive index. With the assumption that the guided lightwave has a slow-varying envelop and is paraxially propagating; i.e., ∂2Ex∂z2<<2n0k∂Ex∂z, ∂2Ey∂z2<<2n0k∂Ey∂z⇒∂2Ex∂z2 and ∂2Ey∂z2 are neglected.

And then we have the following coupled equations

2jn0k⋅∂Ex(x,y,z)∂z=∂∂x1n2(x,y,z)⋅∂∂x[n2(x,y,z)Ex(x,y,z)]+∂2Ex(x,y,z)∂y2

+[n2(x,y,z)−n02]k2Ex(x,y,z)+∂∂x1n2(x,y,z)⋅∂∂y[n2(x,y,z)Ey(x,y,z)]−∂2Ey(x,y,z)∂x∂y

and

2jn0k⋅∂Ey(x,y,z)∂z=∂2Ey(x,y,z)∂x2+∂∂y1n2(x,y,z)⋅∂∂y[n2(x,y,z)Ey(x,y,z)]

+[n2(x,y,z)−n02]k2Ey(x,y,z)+∂∂y1n2(x,y,z)⋅∂∂x[n2(x,y,z)Ex(x,y,z)]−∂2Ex(x,y,z)∂y∂x

These coupled equations are the basic formulas of the full-vector BPM. If we neglect the terms of Axy and Ayx, then the equations are reduced to the formulas of the semi-vector BPM. In case the transverse variation of the refractive index is very small, and that is, 1n2∂∂x(n2Ex)≈∂Ex∂x and 1n2∂∂y(n2Ey)≈∂Ey∂y.

The formulas of the semi-vector BPM can be simplified into the following Fresnel equation, which is the formula of the scalar BPM.

3D Full-vector BPM involving the Runge-Kutta algorithm (by Y. -J. Lin):

The transverse components of the optical field fulfill the following coupled equations

2jn0k⋅∂Ex(x,y,z)∂z=∂∂x1n2(x,y,z)⋅∂∂x[n2(x,y,z)Ex(x,y,z)]+∂2Ex(x,y,z)∂y2

+[n2(x,y,z)−n02]k2Ex(x,y,z)+∂∂x1n2(x,y,z)⋅∂∂y[n2(x,y,z)Ey(x,y,z)]−∂2Ey(x,y,z)∂x∂y

and

2jn0k⋅∂Ey(x,y,z)∂z=∂2Ey(x,y,z)∂x2+∂∂y1n2(x,y,z)⋅∂∂y[n2(x,y,z)Ey(x,y,z)]

+[n2(x,y,z)−n02]k2Ey(x,y,z)+∂∂y1n2(x,y,z)⋅∂∂x[n2(x,y,z)Ex(x,y,z)]−∂2Ex(x,y,z)∂y∂x

According to the Runge-Kutta method, the transverse components of the propagating field are expressed by

Ex(m,n,l+1)=Ex(m,n,l)+K1(m,n,l)+2K2(m,n,l)+2K3(m,n,l)+K4(m,n,l)6Ey(m,n,l+1)=Ey(m,n,l)+J1(m,n,l)+2J2(m,n,l)+2J3(m,n,l)+J4(m,n,l)6,

where

K1m,n,l=Δz2jn0k⋅+−bΔx2Exm,n,l+cΔx2Exm−1,n,l

+1Δy2Exm,n+1,l+−2Δy2Exm,n,l+1Δy2Exm,n−1,l

+n2m,n,l−n02⋅k2⋅Exm,n,l

K2m,n,l=Δz2jn0k⋅

+−bΔx2Exm,n,l+K1m,n,l2+cΔx2Exm−1,n,l+K1m−1,n,l2

+1Δy2Exm,n+1,l+K1m,n+1,l2+−2Δy2Exm,n,l+K1m,n,l2

+1Δy2Exm,n−1,l+K1m,n−1,l2+n2m,n,l−n02⋅k2⋅Exm,n,l+K1m,n,l2

K3m,n,l=Δz2jn0k⋅

+−bΔx2+cΔx2

+1Δy2Exm,n+1,l+K2m,n+1,l2+−2Δy2Exm,n,l+K2m,n,l2

+1Δy2Exm,n−1,l+K2m,n−1,l2

+n2m,n,l−n02⋅k2⋅Exm,n,l+K2m,n,l2

K4m,n,l=Δz2jn0k⋅

+−bΔx2Exm,n,l+K3m,n,l+cΔx2Exm−1,n,l+K3m−1,n,l

+1Δy2Exm,n+1,l+K3m,n+1,l+−2Δy2Exm,n,l+K3m,n,l

+1Δy2Exm,n−1,l+K3m,n−1,l

+n2m,n,l−n02⋅k2⋅Exm,n,l+K3m,n,l

O1m,n,l=Δz8jn0kΔxΔyEym+1,n+1,l⋅n2m+1,n+1,ln2m+1,n,l−1

+Eym,n+1,l⋅−2n2m,n+1,ln2m,n,l+2+Eym−1,n+1,l⋅n2m−1,n+1,ln2m−1,n,l−1

+Eym+1,n−1,l⋅n2m+1,n−1,ln2m+1,n,l−1+Eym,n−1,l⋅−2n2m,n−1,ln2m,n,l+2

+Eym−1,n−1,l⋅n2m−1,n−1,ln2m−1,n,l−1-2Ey(m+1,n,l)+4Ey(m,n,l)-2Ey(m−1,n,l)

O2m,n,l=Δz8jn0kΔxΔyEym+1,n+1,l+O1m+1,n+1,l2⋅n2m+1,n+1,ln2m+1,n,l−1

+Eym,n+1,l+O1m,n+1,l2⋅−2n2m,n+1,ln2m,n,l+2

+Eym−1,n+1,l+O1m−1,n+1,l2⋅n2m−1,n+1,ln2m−1,n,l−1

+Eym+1,n−1,l+O1m+1,n−1,l2⋅n2m+1,n−1,ln2m+1,n,l−1

+Eym,n−1,l+O1m,n−1,l2⋅−2n2m,n−1,ln2m,n,l+2

+Eym−1,n−1,l+O1m−1,n−1,l2⋅n2m−1,n−1,ln2m−1,n,l−1

-2⋅[Ey(m+1,n,l)+O1(m+1,n,l)2]+4⋅[Ey(m,n,l)+O1(m,n,l)2]

-2⋅[Ey(m−1,n,l)+O1(m−1,n,l)2]

O3m,n,l=Δz8jn0kΔxΔyEym+1,n+1,l+O2m+1,n+1,l2⋅n2m+1,n+1,ln2m+1,n,l−1

+Eym,n+1,l+O2m,n+1,l2⋅−2n2m,n+1,ln2m,n,l+2

+Eym−1,n+1,l+O2m−1,n+1,l2⋅n2m−1,n+1,ln2m−1,n,l−1

+Eym+1,n−1,l+O2m+1,n−1,l2⋅n2m+1,n−1,ln2m+1,n,l−1

+Eym,n−1,l+O2m,n−1,l2⋅−2n2m,n−1,ln2m,n,l+2

+Eym−1,n−1,l+O2m−1,n−1,l2⋅n2m−1,n−1,ln2m−1,n,l−1

-2⋅[Ey(m+1,n,l)+O2(m+1,n,l)2]+4⋅[Ey(m,n,l)+O2(m,n,l)2]

-2⋅[Ey(m−1,n,l)+O2(m−1,n,l)2]

O4m,n,l=Δz8jn0kΔxΔyEym+1,n+1,l+O3m+1,n+1,l⋅n2m+1,n+1,ln2m+1,n,l−1

+Eym,n+1,l+O3m,n+1,l⋅−2n2m,n+1,ln2m,n,l+2

+Eym−1,n+1,l+O3m−1,n+1,l⋅n2m−1,n+1,ln2m−1,n,l−1

+Eym+1,n−1,l+O3m+1,n−1,l⋅n2m+1,n−1,ln2m+1,n,l−1

+Eym,n−1,l+O3m,n−1,l⋅−2n2m,n−1,ln2m,n,l+2

+Eym−1,n−1,l+O3m−1,n−1,l⋅n2m−1,n−1,ln2m−1,n,l−1

-2⋅[Ey(m+1,n,l)+O3(m+1,n,l)]+4⋅[Ey(m,n,l)+O3(m,n,l)]

-2⋅[Ey(m−1,n,l)+O3(m−1,n,l)]

J1m,n,l=Δz2jn0k⋅+−2Δx2Eym,n,l

+1Δx2Eym−1,n,l+dΔy2Eym,n+1,l+−eΔy2Eym,n,l+fΔy2Eym,n−1,l

+n2m,n,l−n02⋅k2⋅Eym,n,l

J2m,n,l=Δz2jn0k⋅

+−2Δx2Eym,n,l+J1m,n,l2+1Δx2Eym−1,n,l+J1m−1,n,l2

+dΔy2Eym,n+1,l+J1m,n+1,l2+−eΔy2Eym,n,l+J1m,n,l2

+fΔy2Eym,n−1,l+J1m,n−1,l2

+n2m,n,l−n02⋅k2⋅Eym,n,l+J1m,n,l2

J3m,n,l=Δz2jn0k⋅

+−2Δx2Eym,n,l+J2m,n,l2+1Δx2Eym−1,n,l+J2m−1,n,l2

+dΔy2Eym,n+1,l+J2m,n+1,l2+−eΔy2Eym,n,l+J2m,n,l2

+fΔy2Eym,n−1,l+J2m,n−1,l2+n2m,n,l−n02⋅k2⋅Eym,n,l+J2m,n,l2

J4m,n,l=Δz2jn0k⋅

+−2Δx2Eym,n,l+J3m,n,l+1Δx2Eym−1,n,l+J3m−1,n,l

+dΔy2Eym,n+1,l+J3m,n+1,l+−eΔy2Eym,n,l+J3m,n,l

+fΔy2Eym,n−1,l+J3m,n−1,l

+n2m,n,l−n02⋅k2⋅Eym,n,l+J3m,n,l

P1m,n,l=Δz8jn0kΔxΔyExm+1,n+1,l⋅n2m+1,n+1,ln2m,n+1,l−1

+Exm+1,n,l⋅−2n2m+1,n,ln2m,n,l+2+Exm+1,n−1,l⋅n2m+1,n−1,ln2m,n−1,l−1

+Exm−1,n+1,l⋅n2m−1,n+1,ln2m,n+1,l−1+Exm−1,n,l⋅−2n2m−1,n,ln2m,n,l+2

+Exm−1,n−1,l⋅n2m−1,n−1,ln2m,n−1,l−1-2Ex(m,n+1,l)+4Ex(m,n,l)

-2Ex(m,n−1,l)

P2m,n,l=Δz8jn0kΔxΔyExm+1,n+1,l+P1m+1,n+1,l2⋅n2m+1,n+1,ln2m,n+1,l−1

+Exm+1,n,l+P1m+1,n,l2⋅−2n2m+1,n,ln2m,n,l+2

+Exm+1,n−1,l+P1m+1,n−1,l2⋅n2m+1,n−1,ln2m1,n−1,l−1

+Exm−1,n+1,l+P1m−1,n+1,l2⋅n2m−1,n+1,ln2m,n+1,l−1

+Exm−1,n,l+P1m−1,n,l2⋅−2n2m−1,n,ln2m,n,l+2

+Exm−1,n−1,l+P1m−1,n−1,l2⋅n2m−1,n−1,ln2m,n−1,l−1

-2⋅[Ex(m,n+1,l)+P1(m,n+1,l)2]+4⋅[Ex(m,n,l)+P1(m,n,l)2]

-2⋅[Ex(m,n−1,l)+P1(m,n−1,l)2]

P3m,n,l=Δz8jn0kΔxΔyExm+1,n+1,l+P2m+1,n+1,l2⋅n2m+1,n+1,ln2m,n+1,l−1

+Exm+1,n,l+P2m+1,n,l2⋅−2n2m+1,n,ln2m,n,l+2

+Exm+1,n−1,l+P2m+1,n−1,l2⋅n2m+1,n−1,ln2m1,n−1,l−1

+Exm−1,n+1,l+P2m−1,n+1,l2⋅n2m−1,n+1,ln2m,n+1,l−1

+Exm−1,n,l+P2m−1,n,l2⋅−2n2m−1,n,ln2m,n,l+2

+Exm−1,n−1,l+P2m−1,n−1,l2⋅n2m−1,n−1,ln2m,n−1,l−1

-2⋅[Ex(m,n+1,l)+P2(m,n+1,l)2]+4⋅[Ex(m,n,l)+P2(m,n,l)2]

-2⋅[Ex(m,n−1,l)+P2(m,n−1,l)2]

P4m,n,l=Δz8jn0kΔxΔyExm+1,n+1,l+P3m+1,n+1,l⋅n2m+1,n+1,ln2m,n+1,l−1

+Exm+1,n,l+P3m+1,n,l⋅−2n2m+1,n,ln2m,n,l+2

+Exm+1,n−1,l+P3m+1,n−1,l⋅n2m+1,n−1,ln2m,n−1,l−1

+Exm−1,n+1,l+P3m−1,n+1,l⋅n2m−1,n+1,ln2m,n+1,l−1

+Exm−1,n,l+P3m−1,n,l⋅−2n2m−1,n,ln2m,n,l+2

+Exm−1,n−1,l+P3m−1,n−1,l⋅n2m−1,n−1,ln2m,n−1,l−1

-2⋅[Ex(m,n+1,l)+P3(m,n+1,l)]+4⋅[Ex(m,n,l)+P3(m,n,l)]

-2⋅[Ex(m,n−1,l)+P3(m,n−1,l)]

a=12+n2m+1,n,l2n2m,n,l, b=1+n2m,n,l2⋅1n2m+1,n,l+1n2m−1,n,l,

c=12+n2m−1,n,l2n2m,n,l, d=12+n2(m,n+1,l)2n2(m,n,l),

e=1+n2m,n,l2⋅1n2m,n+1,l+1n2m,n−1,l, f=12+n2m,n−1,l2n2m,n,l

Power distributionE2=(Ex2+Ey2)

3D Semi-vector BPM involving the Runge-Kutta algorithm (by K. -Y. Lee):

The transverse components of the optical field Ex(x,y,z) and Ey(x,y,z) fulfill

2jn0k⋅∂Ex(x,y,z)∂z=∂∂x1n2(x,y,z)⋅∂∂x[n2(x,y,z)Ex(x,y,z)]+∂2Ex(x,y,z)∂y2

+[n2(x,y,z)−n02]k2Ex(x,y,z)

and

2jn0k⋅∂Ey(x,y,z)∂z=∂2Ey(x,y,z)∂x2+∂∂y1n2(x,y,z)⋅∂∂y[n2(x,y,z)Ey(x,y,z)]

+[n2(x,y,z)−n02]k2Ey(x,y,z)

The Runge-Kutta algorithm can be directly to obtain the propagating field components as

Ex(m,i,l+1)=Ex(m,i,l)+K1(m,i,l)+2K2(m,i,l)+2K3(m,i,l)+K4(m,i,l)6

and

Ey(m,i,l+1)=Ey(m,i,l)+J1(m,i,l)+2J2(m,i,l)+2J3(m,i,l)+J4(m,i,l)6,

where

K1(m,i,l)=Δz2jn0k⋅

+Ex(m,i+1,l)−2Ex(m,i,l)+Ex(m,i−1,l)Δy2+[n2(m,i,l)−n02]⋅k2⋅Ex(m,i,l)

K2(m,i,l)=Δz2jn0k⋅

+Ex(m,i+1,l)+K1(m,i+1,l)2−2[Ex(m,i,l)+K1(m,i,l)2]+Ex(m,i−1,l)+K1(m,i−1,l)2Δy2

+[n2(m,i,l)−n02]⋅k2⋅[Ex(m,i,l)+K1(m,i,l)2]

K3(m,i,l)=Δz2jn0k⋅

+Ex(m,i+1,l)+K2(m,i+1,l)2−2[Ex(m,i,l)+K2(m,i,l)2]+Ex(m,i−1,l)+K2(m,i−1,l)2Δy2

+[n2(m,i,l)−n02]⋅k2⋅[Ex(m,i,l)+K2(m,i,l)2]

K4(m,i,l)=Δz2jn0k⋅

+Ex(m,i+1,l)+K3(m,i+1,l)−2[Ex(m,i,l)+K3(m,i,l)]+Ex(m,i−1,l)+K3(m,i−1,l)Δy2

+[n2(m,i,l)−n02]⋅k2⋅[Ex(m,i,l)+K3(m,i,l)]

J1(m,i,l)=Δz2jn0k⋅

+dEy(m,i+1,l)−eEy(m,i,l)+fEy(m,i−1,l)Δy2+[n2(m,i,l)−n02]⋅k2⋅Ey(m,i,l),

J2(m,i,l)=Δz2jn0k⋅

+d[Ey(m,i+1,l)+J1(m,i+1,l)2]−e[Ey(m,i,l)+J1(m,i,l)2]+f[Ey(m,i−1,l)+J1(m,i−1,l)2]Δy2

+[n2(m,i,l)−n02]⋅k2⋅[Ey(m,i,l)+J1(m,i,l)2]

J3(m,i,l)=Δz2jn0k⋅

+d[Ey(m,i+1,l)+J2(m,i+1,l)2]−e[Ey(m,i,l)+J2(m,i,l)2]+f[Ey(m,i−1,l)+J2(m,i−1,l)2]Δy2

+[n2(m,i,l)−n02]⋅k2⋅[Ey(m,i,l)+J2(m,i,l)2]

J4(m,i,l)=Δz2jn0k⋅

+d[Ey(m,i+1,l)+J3(m,i+1,l)]−e[Ey(m,i,l)+J3(m,i,l)]+f[Ey(m,i−1,l)+J3(m,i−1,l)]Δy2

+[n2(m,i,l)−n02]⋅k2⋅[Ey(m,i,l)+J3(m,i,l)]

a=12+n2(m+1,i,l)2n2(m,i,l), b=1+n2(m,i,l)2⋅[1n2(m+1,i,l)+1n2(m−1,i,l)], c=12+n2(m−1,i,l)2n2(m,i,l),

d=12+n2(m,i+1,l)2n2(m,i,l), e=1+n2(m,i,l)2⋅[1n2(m,i+1,l)+1n2(m,i−1,l)], f=12+n2(m,i−1,l)2n2(m,i,l)

Note that coefficients b and e approach 2, but a, c, d, and f approach 1 as the transverse variations of the refractive index profiles are very small. In this case, the above semivectorial formulae for two types of polarizations are reduced into the same scalar expressions.

Power distributionE2=Ex2 or Ey2

3D Scalar BPM involving the Runge-Kutta algorithm (by K. -Y. Lee):

The scalar optical field φ(x,y,z) can be described by the following Fresnel equation

2jn0k⋅∂φ(x,y,z)∂z=∂2φ(x,y,z)∂x2+∂2φ(x,y,z)∂y2+[n2(x,y,z)−n02]k2φ(x,y,z),

Utilizing the finite difference scheme and the Runge-Kutta method, the propagating optical field at (x=mΔx,y=iΔy,z=(l+1)Δz) is expressed by

φ(m,i,l+1)=φ(m,i,l)+K1(m,i,l)+2K2(m,i,l)+2K3(m,i,l)+K4(m,i,l)6,

where

K1(m,i,l)=Δz2jn0k⋅

+φ(m,i+1,l)−2φ(m,i,l)+φ(m,i−1,l)Δy2+[n2(m,i,l)−n02]⋅k2⋅φ(m,i,l)

K2(m,i,l)=Δz2jn0k⋅

+φ(m,i+1,l)+K1(m,i+1,l)2−2φ(m,i,l)−K1(m,i,l)+φ(m,i−1,l)+K1(m,i−1,l)2Δy2

+[n2(m,i,l)−n02]⋅k2⋅[φ(m,i,l)+K1(m,i,l)2]

K3(m,i,l)=Δz2jn0k⋅+φ(m,i+1,l)+K2(m,i+1,l)2−2φ(m,i,l)−K2(m,i,l)+φ(m,i−1,l)+K2(m,i−1,l)2Δy2

+[n2(m,i,l)−n02]⋅k2⋅[φ(m,i,l)+K2(m,i,l)2]

and

K4(m,i,l)=Δz2jn0k⋅

+φ(m,i+1,l)+K3(m,i+1,l)−2φ(m,i,l)−2K3(m,i,l)+φ(m,i−1,l)+K3(m,i−1,l)Δy2

+[n2(m,i,l)−n02]⋅k2⋅[φ(m,i,l)+K3(m,i,l)]

On the other hand, the transparent boundary condition (TBC) is implemented in the computer programming for reducing extra numerical errors due to undesired wave reflections at the edges of computational windows.

Transparent Boundary Conditions (TBC):

Define α+x, α−x, β+x, β−x, α+y, α−y, β+y, and β−y as

α±x+jβ±x≡ln[φ(±M∓1,i,l)φ(±M∓2,i,l)] and α±y+jβ±y≡ln[φ(m,±I∓1,l)φ(m,±I∓2,l)]

TBC shows that

φ(±M,i,l)=φ(±M∓1,i,l)2φ(±M∓2,i,l),α±x<0φ(±M∓1,i,l)⋅exp(jβ±x),α±x≥0

and

φ(m,±I,l)=φ(m,±I∓1,l)2φ(m,±I∓2,l),α±y<0φ(m,±I∓1,l)⋅exp(jβ±y),α±y≥0

Similarly, the values K1(±M,i,l), K2(±M,i,l), K3(±M,i,l), K4(±M,i,l), K1(m,±I,l), K2(m,±I,l), K3(m,±I,l), and K4(m,±I,l) at the edges of the computation windows are obtained by the same rules.

Power distributionφ2

Eg. A laser beam passes an MMI waveguide device. (by K. Huang, 黃建智)

Eg. Simulation of the fundamental mode of a single-mode ridge waveguide.

Eg. Simulation of a 10°-Y-junction 3D waveguide. (by K. –Y. Lee)

Input mode              Output light spots

Eg. For an n×1 MMI optical combiner and a 1×n MMI optical splitter, the BPM simulation shows that Ln×1/L1×nn-1, where Ln×1 is the wide section length of the n×1 MMI optical combiner and L1×n is the wide section length of the 1×n MMI optical splitter, respectively. Consider a SOI waveguide of ncore=3.45, nSiO2=1.5, λ=1.55μm. The thickness of oxide is 2μm. The input single-mode waveguide has a 5μm×5μm cross section.

1×2 MMI splitter     2×1 MMI combiner    1×3 MMI splitter    3×1 MMI combiner 3D BPM simulation (by J. –R. Sze et al.):

| n | 2 | 3 | 4 | 5 |
| --- | --- | --- | --- | --- |
| W | 40μm | 40μm | 40μm | 40μm |
| g | 15μm | 8.333μm | 5μm | 3μm |
| Ln×1 | 1772μm | 2380μm | 2684μm | 2846μm |
| L1×n | 1805μm | 1197μm | 893μm | 731μm |
| Ln×1/L1×n | 0.98≒1 | 1.98≒2 | 3.005≒3 | 3.89≒4 |